DAY2_IMAG_LC500hMULTI_ENG

| ENGLISH | SUBTITLES |
| --- | --- |
| The multi-stage hybrid system |  |
| Realises start-off drive power that exceeds their V8 hybrid model |  |
| While downsizing the engine to a V6. |  |
| It enhances acceleration performance |  |
| With a direct feeling that is unique to hybrid vehicles. |  |
| It also creates a driving rhythm through shift-control by the multi-stage shift device. |  |
| This results in a completely new hybrid system. |  |
| The current Lexus hybrid system, shifts the motor output |  |
| By 2-speeds reduction gear. |  |
| The multi-stage hybrid system is equipped with a multi-stage shift device |  |
| That varies the combined output of the engine and motor. |  |
| This enables the engine power to also be amplified |  |
| Increasing the total torque by 24 percent. |  |
| The multi-stages hybrids multi-stage shift device is arranged in series with the current hybrid system. |  |
| This multi-stage shift device changes the output in four stages. |  |
| This is the engine operating range of the current hybrid |  |
| The multi-stage hybrid changes the output in four stages |  |
| To be able to utilise the engine speeds in wider range from low to high speeds |  |
| Maximum engine output can be used from 50 km/h |  |
| Enhancing the performance of starting acceleration |  |
| Which achieves 0-100 km/h acceleration in 5 seconds. |  |
| Enabling high-speed driving at lower engine RPM also enhances the quietness. |  |
| The maximum speed at which the vehicle can drive with the engine stopped |  |
| Has been increased to 140 km/h. |  |
| The multi-stage hybrid is equipped with 10-speed shift-control |  |
| Multi-stage hybrid removes the rubber-band-feel. |  |
| This provides a direct acceleration feel that follows the increase in engine RPM |  |
| As well as a rhymical shift feeling. |  |
| The all-new multi-stage hybrid system delivers an emotional and exhilarating driving performance. |  |